# APEX PROBLEM SOLVING GUIDE

STEP 1 Replace leadset with known good leadset.

STEP 2 Replace batteries with known good batteries..

STEP 3 Check and make sure that transmitter and central station programmed TTX numbers are correct.

Example:

Transmitter and receiver are both programmable.  If the TTX# on the transmitter is 123 then at the central station you must make sure that you have selected TTX#123 for that sector.

STEP 4 Make sure the sector you are working with is selected and has the patient admitted.

STEP 5 If all above problem solving suggestions have failed, please send transmitter to BMES for repair.

BMES

800-626-4515 Technical Support

8505 Nash rd

Louisville, KY 40214